ENGI 9875 — Special Topics in Embedded Systems

Predictive Maintenance with Anomaly Detection
Using TFLite Micro on NuttX RTOS

Onwukanjo Innocent

Student ID: 202487225

Memorial University of Newfoundland

April 2026

Abstract

This report presents the design and implementation of a predictive maintenance system that detects simulated equipment failure by monitoring motor vibration patterns for anomalies. The system runs on the NuttX real-time operating system targeting the STM32F407 microcontroller, simulated using Renode. A binary classifier neural network was trained offline using Keras and TensorFlow, then quantized to INT8 format and deployed using TensorFlow Lite for Microcontrollers (TFLite Micro). The embedded application is structured as a multi-task pipeline consisting of a high-priority sampling task, a mid-priority inference task running the neural network, and a low-priority alert task. Inter-task communication uses POSIX message queues, shared anomaly logging is protected by counting semaphores, and task notifications wake the alert task upon anomaly detection. The system successfully classifies normal motor vibration (5 Hz sine) and bearing fault signatures (20 Hz whine) with 100% accuracy across test scenarios, demonstrating the feasibility of deploying edge machine learning within a real-time embedded operating system.

# 1. Introduction

## 1.1 Problem Statement

Unplanned equipment failures in industrial settings lead to significant downtime and maintenance costs. Traditional threshold-based monitoring approaches fail to detect subtle degradation patterns that precede catastrophic failure. Machine learning offers a data-driven alternative, but deploying neural network inference on resource-constrained microcontrollers presents unique challenges in memory management, numerical compatibility, and real-time task scheduling.

## 1.2 Motivation

This project explores the intersection of embedded real-time systems and edge machine learning. Rather than streaming sensor data to a cloud server for analysis, the system performs inference directly on the microcontroller, reducing latency and eliminating network dependencies. The NuttX RTOS provides the scheduling and inter-task communication primitives needed to structure the system as a deterministic pipeline.

## 1.3 Objectives

The primary objectives of this project are: (1) train a binary classifier neural network to distinguish normal motor vibration from bearing fault signatures; (2) quantize the model to INT8 format and deploy it using TFLite Micro within NuttX; (3) implement a multi-task pipeline demonstrating priority-based scheduling, message queues, semaphore-protected shared resources, and task notifications; and (4) validate the complete system in the Renode simulator targeting the STM32F407 platform.

## 1.4 Learning Outcomes Addressed

LO1 (Real-Time Scheduling): Rate-monotonic priority assignment is applied across three tasks with distinct timing requirements. LO3 (Concurrency Issues): A shared anomaly event log accessed by multiple tasks demonstrates the need for semaphore-based mutual exclusion. LO5 (Device Drivers): While a full character device driver was not implemented due to the complexity of the TFLite Micro integration, the sampling task simulates the driver interface pattern of periodic sensor reads feeding an inter-task communication channel.

# 2. Background

## 2.1 Real-Time Operating Systems

A real-time operating system (RTOS) provides deterministic scheduling guarantees essential for embedded control applications [1]. NuttX is a POSIX-compliant RTOS that supports preemptive priority-based scheduling, POSIX threads, message queues, and semaphores [2]. Its small footprint and standards compliance make it suitable for microcontroller targets including the ARM Cortex-M4 family.

## 2.2 Rate-Monotonic Scheduling

Rate-Monotonic Scheduling (RMS) is an optimal fixed-priority assignment policy for periodic tasks: shorter periods receive higher priorities [3]. A set of n periodic tasks is schedulable under RMS if the total processor utilization U satisfies U ≤ n(2^(1/n) – 1). For three tasks, this bound is approximately 0.780. RMS provides a theoretical framework for verifying that all task deadlines will be met.

## 2.3 TensorFlow Lite for Microcontrollers

TensorFlow Lite for Microcontrollers (TFLite Micro) is a stripped-down inference engine designed for devices with as little as 16 KB of RAM [4]. It supports INT8 quantized models, eliminating the need for floating-point hardware. The framework uses a statically allocated tensor arena and a FlatBuffer model format. Deploying TFLite Micro on NuttX requires resolving namespace incompatibilities between the C++ standard library expectations of TFLite and NuttX’s minimal C math library.

## 2.4 Anomaly Detection in Predictive Maintenance

Predictive maintenance uses sensor data to forecast equipment failure before it occurs [5]. Vibration analysis is a primary technique for rotating machinery, where frequency-domain changes indicate bearing degradation [6]. Binary classifiers trained on labeled vibration data can distinguish normal operating signatures from fault conditions. INT8 quantization enables deployment of these classifiers on microcontrollers with minimal accuracy loss [7].

## 2.5 Renode Simulation

Renode is an open-source simulation framework for embedded systems that provides cycle-accurate emulation of ARM Cortex-M processors [8]. It enables full-system testing without physical hardware, including UART output for console interaction. The STM32F4 Discovery board platform is directly supported.

# 3. System Design

## 3.1 System Architecture

The system is structured as a three-stage pipeline running on NuttX. Each stage is implemented as a POSIX thread with explicit priority assignment:

| Task | Priority | Responsibility |
| --- | --- | --- |
| Sampling Task | 110 (highest) | Periodic vibration data generation at 100 Hz, quantization to INT8, transmission via message queue |
| Inference Task | 100 (mid) | Receives batches from queue, runs TFLite Micro inference, logs results, signals alert task on anomaly |
| Alert Task | 90 (lowest) | Blocks on semaphore, wakes on anomaly notification, reads shared log, issues maintenance alerts |

Table 1: Task decomposition with priority assignments following Rate-Monotonic principles.

## 3.2 Inter-Task Communication

Data flows through the pipeline using two distinct mechanisms. POSIX message queues transfer sensor batches from the sampling task to the inference task. The queue is configured with a capacity of 5 messages, each containing a sensor_batch_t structure with 10 INT8 quantized readings, a batch identifier, and quantization parameters. This decouples the producer and consumer, allowing the sampling task to continue reading at its fixed rate even if inference takes longer for a particular batch.

A binary semaphore (g_alert_sem) implements the task notification pattern. The alert task blocks on sem_wait(), consuming no CPU cycles while idle. When the inference task detects an anomaly, it calls sem_post() to wake the alert task directly, avoiding the overhead and non-determinism of polling.

## 3.3 Concurrency Management

The anomaly event log (g_anomaly_log) is a shared data structure accessed by both the inference task (writing new entries) and the alert task (reading the latest entry for alert generation). A counting semaphore (g_log_sem) initialized to 1 protects this shared resource. Both tasks acquire the semaphore before accessing the log and release it immediately after, minimizing the critical section duration. This prevents data corruption that would occur if the alert task read a partially-written entry.

## 3.4 Scheduling Analysis

Applying Rate-Monotonic Analysis to the three-task system:

| Task | Period (Ti) | Exec Time (Ci) | Utilization |
| --- | --- | --- | --- |
| Sampling | 100 ms | ~2 ms | 0.020 |
| Inference | 100 ms | ~15 ms | 0.150 |
| Alert | Aperiodic | ~1 ms | ~0.010 |

Table 2: Rate-Monotonic Scheduling analysis parameters.

The total utilization U = 0.020 + 0.150 + 0.010 = 0.180, which is well below the RMS bound of 0.780 for three tasks. The system is therefore schedulable with significant margin. The alert task is aperiodic (event-driven via semaphore), so its utilization contribution is averaged over the observation period.

# 4. Implementation

## 4.1 Hardware and Software Stack

The target platform is the STM32F407VG microcontroller (ARM Cortex-M4, 1 MB Flash, 192 KB SRAM) on the STM32F4 Discovery board. Development and testing were conducted entirely in the Renode simulator. The software stack consists of NuttX RTOS (latest mainline), TFLite Micro (commit cfa4c91d), the arm-none-eabi-gcc cross-compiler toolchain, and Python 3 with TensorFlow/Keras for offline model training.

## 4.2 Model Training and Quantization

The neural network is a binary classifier with a single hidden layer of 16 neurons (ReLU activation) and a single sigmoid output. Training data consists of 4000 batches of 10 time-domain samples each: 2000 normal samples (5 Hz sine wave with Gaussian noise, σ=0.1) and 2000 fault samples (20 Hz sine wave with noise). The model was trained for 15 epochs using the Adam optimizer with binary cross-entropy loss.

Post-training INT8 quantization was applied using the TensorFlow Lite converter with a representative dataset of 500 samples. Input and output types were set to int8, targeting the TFLITE_BUILTINS_INT8 op set. The resulting model is 2,608 bytes, small enough to embed directly in flash memory as a C header file generated by xxd.

## 4.3 TFLite Micro Integration Challenges

Integrating TFLite Micro with NuttX required resolving a fundamental incompatibility: TFLite’s C++ source code assumes a full ISO C++ standard library where math functions like std::round, std::sqrt, std::pow, std::exp, std::isnan, and std::signbit are available in the std namespace. NuttX provides only a C math library (math.h) where these functions exist in the global namespace without float-specific overloads.

The solution was a compatibility shim header (nuttx_math_shim.h) that: (1) imports all required C math functions into the std namespace using using-declarations; (2) provides inline float overloads (e.g., float std::sqrt(float) calling sqrtf()) to resolve function pointer type mismatches where TFLite passes std::sqrt as a float(*)(float) argument; (3) handles macro-defined functions (isnan, isinf, signbit) which cannot use using-declarations by wrapping them in template functions; and (4) adds a template overload for pow to resolve ambiguity when called with integer arguments. This shim is force-included via the -include compiler flag in both the TFLite Micro library Makefile and the application Makefile.

## 4.4 NuttX Application Structure

The application is registered as a NuttX built-in command (pred_main_ml) with the entry point pred_main_ml_main() using extern "C" linkage. The main function initializes shared state (semaphores, message queue), then creates three pthreads with explicit priority and stack size assignments. The inference task receives an 8 KB stack to accommodate the TFLite tensor arena, while the sampling and alert tasks use 4 KB each.

The NuttX configuration enables CONFIG_TFLITEMICRO and CONFIG_TFLITEMICRO_DEBUG (required to link the MicroPrintf symbol), along with POSIX message queues (CONFIG_MQ) and pthreads (CONFIG_PTHREAD). The build system uses the standard NuttX apps framework with Make.defs for Kconfig integration and Application.mk for compilation rules.

## 4.5 Message Queue Configuration

The POSIX message queue /ml_vib_queue is created with mq_open() using O_CREAT|O_RDWR and configured for a maximum of 5 pending messages. Each message is a sensor_batch_t structure containing 10 INT8 quantized readings, a batch identifier, and the input quantization parameters (scale and zero point). The sampling task opens the queue with O_WRONLY and the inference task with O_RDONLY, establishing a unidirectional producer-consumer channel.

# 5. Evaluation

## 5.1 Functional Verification

The system was tested in Renode with 20 inference batches: 10 normal (5 Hz sine) followed by 10 fault (20 Hz whine). Results demonstrate 100% classification accuracy:

| Batch Range | Signal Type | Expected | Actual | Accuracy |
| --- | --- | --- | --- | --- |
| 0–9 | 5 Hz sine | NORMAL | 10/10 OK | 100% |
| 10–19 | 20 Hz whine | FAULT | 10/10 FAULT | 100% |

Table 3: Classification results across 20 test batches.

## 5.2 Memory Usage

The system’s memory footprint is summarized below:

| Resource | Usage |
| --- | --- |
| Flash (total NuttX image) | 138,332 bytes (13.2% of 1 MB) |
| SRAM (static allocation) | 17,336 bytes (15.1% of 112 KB) |
| Model size (in flash) | 2,608 bytes |
| Tensor arena (in SRAM) | 708 bytes used of 8,192 allocated |

Table 4: Memory usage breakdown.

The tensor arena uses only 708 bytes of the 8,192 allocated, indicating the model is well within the SRAM budget. The 8 KB allocation could be reduced to 2 KB for this specific model, freeing memory for additional application logic.

## 5.3 Task Scheduling Validation

The Renode output confirms correct task scheduling behavior. The sampling task (priority 110) consistently produces batches before the inference task (priority 100) consumes them, with the message queue buffering the pipeline. The alert task (priority 90) only executes when woken by the inference task’s semaphore post, demonstrating the event-driven notification pattern. No priority inversion was observed, as the critical sections protecting the anomaly log are short (single struct writes/reads).

## 5.4 Comparison with Alternatives

Compared to cloud-based inference, the edge approach eliminates network latency entirely — inference completes within the same 100 ms sampling window. Compared to threshold-based anomaly detection (as implemented in the earlier pred_main.c version using variance calculation), the neural network approach provides a learned decision boundary that can generalize to more complex fault patterns without manual threshold tuning.

# 6. Discussion

## 6.1 Challenges Faced

The most significant challenge was integrating TFLite Micro with NuttX’s C++ environment. NuttX’s cmath header does not inject C math functions into the std namespace, which is a hard requirement of TFLite’s internal headers. Resolving this required iterative development of a compatibility shim that handles: (1) missing std:: namespace declarations for round, pow, sqrt, exp, log, and other functions; (2) float overloads needed for function pointer type matching; (3) macro-vs-function differences for isnan/isinf/signbit; (4) overload ambiguity when calling functions like pow(int, int) with both float and double candidates; and (5) the abs function requiring int, float, double, long, and long long overloads to avoid ambiguity with stdlib’s int abs(int).

A secondary challenge was the linker error for MicroPrintf, which was silently stripped by the TF_LITE_STRIP_ERROR_STRINGS preprocessor flag in non-debug builds. Enabling CONFIG_TFLITEMICRO_DEBUG resolved this.

## 6.2 Lessons Learned

First, embedded C++ is not desktop C++. The assumption that #include <cmath> provides std::round is only valid with a full standard library implementation. Embedded RTOSes often provide minimal C library wrappers. Second, the -include compiler flag is a powerful mechanism for injecting compatibility shims without modifying third-party source code. Third, INT8 quantization is remarkably effective for simple classifiers: the 2,608-byte model achieves perfect accuracy on the test scenarios while using under 1 KB of runtime memory.

## 6.3 Learning Outcomes Reflection

LO1 (Real-Time Scheduling): The three-task pipeline with explicit priority assignment directly demonstrates rate-monotonic scheduling principles. The schedulability analysis confirms the system meets all timing constraints with significant utilization margin (0.180 vs. the 0.780 bound).

LO3 (Concurrency Issues): The shared anomaly log provides a concrete scenario where unsynchronized access would cause data corruption. The semaphore-based solution demonstrates mutual exclusion in practice.

LO5 (Device Drivers): While a formal NuttX character device driver was not implemented, the sampling task’s architecture mirrors the driver pattern: periodic reads from a data source, packaging into structured messages, and delivery via an inter-task channel. This represents the application-level interface that would sit above a character device in a production system.

## 6.4 RTOS Evaluation

NuttX proved well-suited for this project due to its POSIX compliance, which allowed using standard pthread, mqueue, and semaphore APIs without learning proprietary interfaces. The NuttX apps build system integrates cleanly with third-party C++ libraries via Makefile variables (CXXFLAGS, COMMON_FLAGS). The main limitation encountered was the incomplete C++ standard library, which required the math shim workaround.

# 7. Conclusion

## 7.1 Summary

This project successfully demonstrates a predictive maintenance system combining edge machine learning with real-time embedded task management. A TensorFlow Lite Micro INT8 classifier running on NuttX correctly distinguishes normal motor vibration from bearing fault signatures across all test scenarios. The multi-task architecture uses POSIX message queues, semaphore-protected shared state, and event-driven task notifications to implement a deterministic inference pipeline.

## 7.2 Future Work

Several enhancements would strengthen the system: (1) implementing a full NuttX character device driver (/dev/accel0) for the simulated accelerometer, completing the LO5 objective; (2) replacing the binary classifier with an autoencoder for unsupervised anomaly detection, eliminating the need for labeled fault data; (3) deploying on physical STM32F4 hardware with a real MEMS accelerometer; (4) adding frequency-domain features (FFT) to improve fault discrimination; and (5) integrating the PLG observability stack for fleet-wide equipment monitoring.

## 7.3 AI Tool Usage Statement

Claude (Anthropic) was used as a development assistant for: debugging TFLite Micro build errors and developing the nuttx_math_shim.h compatibility layer; structuring the multi-task application architecture; and drafting this report. All architectural decisions, model training, NuttX configuration, and Renode testing were performed by the author. External code (TFLite Micro, NuttX) is attributed in the build system and README.

# References

- J. W. S. Liu, Real-Time Systems, Prentice Hall, 2000.

- G. Nutt, "NuttX: A Mature, Real-Time Embedded Operating System," nuttx.apache.org, 2024.

- C. L. Liu and J. W. Layland, "Scheduling Algorithms for Multiprogramming in a Hard-Real-Time Environment," Journal of the ACM, vol. 20, no. 1, pp. 46–61, 1973.

- R. David et al., "TensorFlow Lite Micro: Embedded Machine Learning for TinyML Systems," Proceedings of Machine Learning and Systems (MLSys), vol. 3, 2021.

- A. K. S. Jardine, D. Lin, and D. Banjevic, "A Review on Machinery Diagnostics and Prognostics Implementing Condition-Based Maintenance," Mechanical Systems and Signal Processing, vol. 20, no. 7, pp. 1483–1510, 2006.

- R. B. Randall and J. Antoni, "Rolling Element Bearing Diagnostics — A Tutorial," Mechanical Systems and Signal Processing, vol. 25, no. 2, pp. 485–520, 2011.

- B. Jacob et al., "Quantization and Training of Neural Networks for Efficient Integer-Arithmetic-Only Inference," Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR), pp. 2704–2713, 2018.

- Antmicro, "Renode: Open Source Simulation Framework," renode.io, 2024.

- IEEE, "IEEE Std 1003.1-2017 — POSIX," The Open Group Base Specifications Issue 7, 2018.

- A. S. Tanenbaum, Modern Operating Systems, 4th ed., Pearson, 2014.

- Y. Cheng et al., "A Survey of Model Compression and Acceleration for Deep Neural Networks," IEEE Signal Processing Magazine, 2017.

- M. Verucchi et al., "Real-Time Clustering for Edge AI: TinyML Approaches for Predictive Maintenance," IEEE Access, vol. 8, pp. 150396–150407, 2020.